PRICE SCHEDULE FOR THE CONSTRUCTION OF 2- STORIED(PHASE -1, G.F,EXCEPT STAIR BLOCK & INTERNAL ELECTRIFICATION) S.D.P.O”S OFFICE BUILDING AT MALBAZAR, DIST.JALPAIGURI

| S.No. | Description of item | Quantity | Unit | Rate (Rs.) | Amount (Rs.) |
| --- | --- | --- | --- | --- | --- |
| 1 | Earth work in excavation of foundation trenches or drains in all sorts of soil (including mixed soil but excluding laterite or sandstone) including removing spreading or stacking the spoils within a lead of 75m. as directed. The item includes necessary trimming the sides of trenches, leveling, dressing and ramming the bottom, bailing out water as required complete. - Depth of excavation not exceeding 1500 mm. | 90 | %M3 | 7377.04 | 6639.00 |
| 2 | Earthwork in filling in foundation trenches or plinth with good earth in layers not exceeding 150 mm including watering and ramming etc. layer by layer complete. - With earth obtained from excavation of foundation. - (iii) With carried earth arranged by the contractor within a radius exceeding 3 km but not exceeding 5 km including the cost of carried earth | 65 70 | %M3 %M3 | 4759.52 30906.91 | 3094.00 21635.00 |
| 3 | Single brick flat soling of picked jhama bricks including ramming and dressing bed to proper level and filling joints with powdered earth or local sand. | 85 | M2 | 237.87 | 20219.00 |
| 4 | Ordinary Cement Concrete (mix 3:1.5:1) M-20 with graded stone chips (20 mm down) excluding shuttering and reinforcement, if any. In ground floor as per IS 456-2000. a) River Bajree a) In ground floor. | 55 | M3 | 4522.55 | 248740.00 |
| 5. | Ordinary Cement Concrete (mix 1;2:4) with graded stone chips (20 mm size) excluding shuttering and reinforcement, if any. b) River Bazree. a) In ground floor | 3 | M3 | 3783.62 | 11351.00 |
| 6. | (II) Cement concrete with 20 mm down graded bazree excluding shuttering. N.B Varity In ground floor - 6:3:1 proportion | 10 | M3 | 3072.25 | 30723.00 |
| 7. | Hire and labour charges for shuttering with centering and necessary staging upto 4 m using approved stout props and thick hard wood planks of approved thickness with required bracing for concrete slabs, beams, columns, lintels curved or straight including fitting, fixing and striking out after completion of works. I) Upto roof of ground floor - 25 mm to 30 mm thick wooden shuttering as per decision & direction of the EIC.. - Steel shuttering or 9 to 12mm thick approved quality ply board shuttering in any concrete work - 25 mm to 30 mm shuttering without staging in foundation etc. | 350 60 100 | M2 M2 M2 | 257.57 303.85 163.52 | 90150.00 18231.00 16352.00 |
| 8. | Reinforcement for reinforced concrete work in all sorts of structures including distribution bars, stirrups, binders etc. including supply of rods, initial straightening and removal of loose rust (if necessary), cutting to requisite length, hooking and bending to correct shape, placing in proper position and binding with 16 gauge black annealed wire at every inter-section, complete as per drawing and direction. - Tor steel/Mild steel. - For works in foundation basement and upto roof of ground floor/upto 4m. | 5.75 | MT | 55954.40 | 321738.00 |
| 9. | Brick work with Ist class bricks in cement mortar (4:1). - In foundation and plinth. - In superstructure. (i) Ground floor. | 15 1 | M3 M3 | 4067.59 4215.58 | 61014.00 4216.00 |
| 10. | 125mm thick brick work with Ist class bricks in cement mortar (4:1). - In ground floor. | 175 | M2 | 520.79 | 91138.00 |
| 11. | - Marble flooring 15 to 18 mm thick tiles set in floor,lobby,stair with cement mortar (2:1) including leveling Course with same mortar as required and high polishing (marble other than Italian) as per direction of Enginer-in –charge in Ground floor. (a)with makrana white/ Adranga/MakranaPlain pink/ Makrana Adranga pink/Garbh gulabi /Udaypur pink/Udaypur green/black Bhaslana). (ii) Area of each tiles exceeding 0.30 Sqm but not exceeding 0.6 Sqm. | 15 | M2 | 1550.18 | 23253.00 |
| 12. | Supplying, fitting and fixing 18 mm to 22 mm thick kota stone slab in wall, dado in cement mortar (1:3) including making suitable arrangement to hold the stone properly by brass/copper hooks including pointing in cement mortar (1:2) (1 cement: 2 stone dust) with admixture of pigments matching the stone shade, including grinding and polishing all complete as per direction of EIC including cost of materials, labour, scaffolding, staging, curing complete. | 45 | M2 | 957.30 | 43079.00 |
| 13. | 25 mm thick damp proof course with cement concrete (1:1.5:3) (with graded stone aggregate 10 mm Nominal size) and painting the top surface with a coat of bitumen using 1.7 kg per sqm including heating the bitumen and cost and carriage of all materials complete. | 8 | M2 | … 245.44 | 1964.00 |
| 14. | Supplying, fitting and fixing 1st quality Ceramic tiles of any size in wall and floors including borders to match with the existing work, set in sand cement mortar (4:1) including backing concrete (1:2:4) & 4 nos. of key stones (10 mm) fixed with araldite at the back of each tile & finishing the joints with white cement mixed with colouring oxide (if required) to match the colour of the tiles.(B) ii) Colour Decorative | 35 | M2 | 955.90 | 33457.00 |
| 15. | Supplying fitting & fixing Cast iron down pipes of approved quality in position with necessary clamps nails including making hole in wall , floor etc and cutting trenches in any soil through masonary concrete structured. If necessary and mending good damages including jointing with jointing materials complete.. ii) 100 mm dia.(External) | 10 | M | 643.59 | 6436.00 |
| 16. | Supplying fitting fixing fan hook for ceiling with 1 m long 16 mm dia rod complete including mending damages. | 10 | No. | 70.00 | 700.00 |
| 17. | Supplying & laying true to line and level vitrified tiles of approved brand (size not less than 600x 600mmx10 mm thick) in floor, skirting etc. set in 20 mm sand cement mortar (1:4) & 2mm thick cement slurry back side of tiles using cement @ 2.91 Kg/m2 or using polymerized adhesive (6mm thick layer applied) laid after application slurry using 1.75 kg of cement per m2 below mortar only, joints grouted with polishing the tiles using soft & dry cloth upto mirror finish complete i/c the cost of materials, grout materials to be supplied by contractor). - With application slurry @ 1.75 Kg /sqm, 20mm sand cement mortar (1:4)& 2mmthick cement slurry at back side of tiles, 0.2 Kg/ Sqm. White cement for joint filling with pigment. - Light Colour. | 65 | M2 | 1066.45 | 69319.00 |
| 18. | (a) M.S. or W.I. ornamental grill of approved design joints continuously welded with M.S. or W.I. flats and bars for windows, railing etc. fitted and fixed with necessary screws and lugs in ground floor ii) Grill weighing above 10 kg. / Sqm. and upto 16 kg. / Sqm. | 3.0 | Qtl. | 6382.19 | 19147.00 |
| 19. | ii)Godrej Hydralic door closer fitted & fixed complete. - Heavy type. | 1 | Each | 2150.70 | 2150.00 |
| 20. | Supplying, fitting and fixing windows and ventilators with or without integrated grills conforming to I.S 1038-1975 and manufactured from rolled steel sections conforming to I.S. 7452-1974 with non-friction projecting type, to match with existing surface complete in all respect excluding glazing. (b) Openable steel windows as per I.S. sizes with side hung shutters and horizontal glazing bars, with or without fixed type ventilators. iv) Two point nose steel handle including fitting and fixing. vi) Steel peg stay 300mm long including fitting and fixing. | 25 20 20 | M2 No. No. | 1552.37 27.00 20.0 | 38809.00 540.00 400.00 |
| 21. | Supplying best Indian sheet glass panes set in putty and fitted and fixed with nails & putty complete in all floors iii) 3mm thick | 20 | M2 | 952.13 | 19043.00 |
| 22. | Woodwork in door and window frame fitted and fixed complete including a protective coat of painting at the contact surface of the frame. b) Sal: Siliguri. | 0.25 | M3 | 112006.00 | 28002.00 |
| 23. | b) Supplying `Godrej` mortice lock chromium plated with keys 6 levers including fitting & fixing complete. | 1 | Each | 1668.81 | 1669.00 |
| 24. | M.S. clamp for fixing door and window frame made of flat bent bar, end bifurcated, fixed in cement concrete with stone chips (4:2:1) fitted and fixed complete as per direction. c) 40 mm x 6 mm , 250 mm length. | 60 | No. | 34.30 | 2058.00 |
| 25. | Iron but hinges of approved quality fitted and fixed with steel screws, with ISI mark. viii) 100 mm x 58 mm x 1.90 mm. | 70 | No. | 45.73 | 3201.00 |
| 26. | Decorative cement base paint of approved quality after scraping the surface thoroughly as necessary on plastered, ruled or concreted surface .a) Two coats. | 200 | M2 | 39.74 | 7948.00 |
| 27. | Applying Interior grade Acrylic primer on plastered surface to receive flat wall paint or plastic emulsion paint including preparing putty etc. a) One coat | 300 | %M2 | 2545.20 | 7636.00 |
| 28. | Anodized aluminum skeleton bolt of approved quality manufactured from extruded section conforming to I.S. specification (I.S.204/74) fitted and fixed with cadmium-plated screws. iii) 300 mm long x 12 mm dia rod. | 10 | No. | 127.86 | 1279.00 |
| 29. | Anodised aluminium Aldrop/Sliding bolts of approved quality manufactured from extruded section conforming to I.S. Specification (I.S. 2681/66) fitted and fixed complete. - 300 mm x 19 mm dia bolt. | 5 | No. | 332.01 | 1660.00 |
| 30. | Anodized aluminium D type handle of approved quality manufactured from extruded section conforming to I.S. specification (I.S 230/72) fitted and fixed complete. a) With continuous plate base (hexagonal/round rod).vi) 150 mm grip x 12 mm dia rod. | 10 | No. | 118.76 | 1188.00 |
| 31. | Panel shutters of door and window, as per design (each panel consisting of single plank without joint), including fitting and fixing the same in position but excluding the cost of hinge and other fittings. iii) 35 mm thick shutters with 19 mm thick panel. - Champ , Sisu | 11 | M2 | 2349.36 | 25843.00 |
| 32. | Supplying fitting and fixing Fiber reinforced polymer (FRP) Composite door frame as per approved section with glass fiber reinforced plastic molded skins and a special sandwich core, so as to impart monolithic composite structure as per approved technology of Department of science and technology (DST) to satisfy IS: 4020 door testing performance criteria. - 66mmx90mm | 20 | M | 571.93 | 11439.00 |
| 33. | Supplying fitting and fixing Fiber reinforced polymer (FRP) Composite door shutter as per approved design with glass fiber reinforced plastic molded skins and a special sandwich core, so as to impart monolithic composite Department of science and technology (DST) to satisfy IS: 4020 door testing performance criteria ii) 32 mm thick | 5.2 | M2 | 3200.48 | 16643.00 |
| 34. | Plaster (to wall, floor, ceiling etc.) with sand and cement mortar including rounding off or chamfering corners as directed and making out joints or roughening of concrete surface, including throating, nosing and drip course where necessary. i) With 4:1 cement mortar. In ground floor. a) 20 mm thick plaster. b) 15 mm thick plaster.. c) 10 mm thick plaster. | 170 400 100 | M2 M2 M2 | 140.85 119.03 92.92 | 23945.00 47612.00 9292.00 |
| 35. | Neat cement punning about 1.5 mm. thick in wall, dado, window, sills, floor, drain etc. | 100 | M2 | 28.51 | 2851.00 |
| 36. | Lime punning about 3mm thick with shell lime and stone lime (2:1) | 400 | M2 | 58.50 | 23400.00 |
| 37. | Applying cement primer of approved quality brand on plastered or concrete surface old or new to receive cement base paint including scrapping and preparing the surface thoroughly, complete as per direction of EIC. - In ground floor. | 25 | M2 | 14.66 | 367.00 |
| 38. | Applying Acrylic Emulsion Paint of approved make and brand on walls and ceiling including sand papering intermediate coats including putty . Two coats ii) Luxury Quality. | 300 | M2 | 56.26 | 16878.00 |
| 39. | Protective and decorative textured 100%Premium Acrylic finish Exterior paint of approved quality as per Manufacturer’s specification and as per direction of E-in-C to be applied over a coat of cement based primer Required. The rates includes the cost of materials, Labour, scaffolding and all others incidental charges But excluding the cost of one coat of cement based Primer required for new cement plastered surface. i) Two coat | 15 | M2 | 92.21 | 1383.00 |
| 40. | Priming one coat on timber, plastered or on steel or other metal surface with synthetic enamel/oil bound primer of approved quality including smoothening surface by sand papering etc. | 135 | M2 | 28.38 | 3831.00 |
| 41. | Painting with best quality synthetic enamel paint of approved make and brand including smoothening surface by sand papering etc. including using of approved putty etc. on the surface, if necessary. a) On timber or plastered surface. With super gloss (hi-gloss) ii) Two coats (with any shade except white). b) On steel or other metal surface. ii) Two coats (with any shade except white). | 50 85 | M2 M2 | 62.62 59.99 | 3131.00 5099.00 |
| 42 | Supplying and laying chequred tile of any shade & of approved quality with (1:1.5:3) cement concrete laid in panels or patterns as directed in pavement. Footpath etc. Including necessary underlay lime /cement mortar (3:1) complete in all respect with all labor and materials - 30mm thick red /variety. | 2 | M2 | 458.41 | 917.00 |
|  | Total for (A) Building Works |  |  |  | 1450809=00 |
| # (B) Sanitary & Plumbing Works |  |  |  |  |  |
| 43. | Supplying, fitting and fixing G.I. pipes of TATA make with all necessary accessories, specials viz. socket, bend, tee, union, cross, elbo, nipple, long screw, reducing socket, reducing tee, short piece etc. fitted with holder bats clamps, including cutting pipes, making threads, fitting, fixing etc. complete in all respect including cost of all necessary fittings as required, jointing materials and two coats of painting with approved paint in any position above ground. A) For Exposed Work. a) (ii) 15 mm dia medium quality. c) (ii) 25 mm dia medium quality. d) (ii) 32 mm dia medium quality. e) (ii) 50 mm dia medium quality | 20 20 12 3 | M M M M | 194.00 300.00 343.00 520.00 | 3880.00 6000.00 4116.00 1560.00 |
| 44. | Supplying, fitting and fixing gunmetal wheel valve of approved brand and make tested to 21 Kg per sq cm. v) 32 mm. vii) 25 mm. | 1 2 | No. No. | 1015.00 708.00 | 1015.00 1416.00 |
| 45. | Anglo Indian W.C. in white glazed vitreous china ware of approved make supplied fitted and fixed complete in position with necessary bolts, nuts etc. b) with S trap ( with vent) | 2 | No. | 3126.00 | 6252.00 |
| 46. | Supplying fitting and fixing with cement jointing (3:1) salt glazed stoneware pipe including excavation of earth upto 1.50 metre depth in all sorts of soil both mixed or unmixed and refilling ( but excluding concreting at bottom and sides).iii) 150mm | 15 | No. | 300.00 | 4500.00 |
| 47. | Squating plate with integral flushing in white vitreous chinaware of approved make supplied ,fitted and fixed in cement concrete(6:3:1) with jhama chips complete. 450 mmX 350 mm | 1 | Each | 1049.00 | 1049.00 |
| 48. | Closet seat of approved make with lid and c.p hinges, rubber buffer and brass screws supplied fitted and fixed complete. - Anglo Indian i) Plastic( hallow type) white | 2 | Each | 405.00 | 810.00 |
| 49. | Supplying fitting and fixing Liquid soap holder. a) Cromium plated. | 3 | Each | 428.00 | 1284.00 |
| 50 | Wash Basins complete with pillar tap waste nozzle, jam nut and nipple & nut supplied, fitted and fixed including cost of jointing materials. | 3 | No. | 252.00 | 756.00 |
| 51 | Supplying fitting and fixing bib cock or stop cock. (c)(ii) Chromium plated. Bib cock with wall flange.delux (heavy type) d) ii) Chromium plated Concealed stop cock heavy duty | 6 8 | No. No | 916.00 798.00 | 5496.00 6384.00 |
| 52 | Supplying plastic water storage tank of approved quality with closed top with lid (Black) Multilayer. d) 1000 litre capacity. | 1 | No. | 5991.00 | 11961.00 |
| 53 | Supplying, fitting and fixing 10-litre P.V.C. low-down cistern conforming to I.S. specifications with P.V.C. fittings complete, C.I. brackets including two coats of painting to bracket etc. | 3 | No. | 1129.00 | 3387.00 |
| 54. | Supplying fitting and fixing glass shelf with Aluminum guard rails. - Ordinary type with 5.5 mm sheet glass i) 600 mmX 125 mm | 3 | Each | 370.00 | 1110.00 |
| 55 | Wash Basin of white vitreous chinaware of approved make (without fittings) supplied, fitted and fixed in position on 75 mm X 75 mm X 75 mm wood blocks and C.I. brackets including two coats of painting of C.I. brackets. ii) 630 mm X 450 mm size. | 3 | No. | 1425.00 | 4275.00 |
| 56 | Supplying, fitting and fixing pedestal of approved make for washbasin.(white) | 3 | No. | 1266.00 | 3798.00 |
| 57 | Supplying, fitting and fixing pillar cock of approved make. c) C.P. Pillar Cock -15 mm.(Jaguar or equivalent) | 3 | N0 | 885.00 | 2655.00 |
| 58 | Cast iron soil pipe only conforming to I.S. 3989 / 1970 and I.S. 1729 / 1964 with bobbins, nails etc. including making holes in the wall, floor etc. and cutting trenches etc. in any floor mending good damages with necessary jointing materials and painting two coats to the exposed surface with approved paint complete. a) With valamoid joints including sealing with sand cement mortar (4:1) upto quarter depth. - 100 mm dia (internal). | 12 | M | 712.00 | 8544.00 |
| 59. | Supplying fitting and fixing cast iron extension pipes for using in Traps only ( connector) conforming to I.S.3989/1970 and 1729/1964 including lead caulked joints and painting two coats to the exposed surface complete. v) 100mm x 450mm vii) 100mm x 600mm | 5 5 | No. No. | 586.00 663.00 | 2930.00 3315.00 |
| 60. | Cast iron single branch equal with door conforming to I.S. 1729 / 1970 including jointing and painting two coats to the exposed surface. a) With valamoid joints including sealing the top with cement mortar (4:1). ii) 100 mm dia. | 5 | No. | 915.00 | 4575.00 |
| 61. | Cast iron double branch equal with door conforming to I.S. 1729 / 1970 including jointing and painting two coats to the exposed surface complete. b) With valamoid joints including sealing the top with cement mortar (4:1). ii) 100 mm dia | 3 | No. | 565.65 | 1697.00 |
| 62 | H.C.I. bend without door conforming to I.S.S. including jointing complete. a) With valamoid joints including sealing the top with cement mortar (4:1). ii) 100 mm dia. | 4 | No. | 432.00 | 1728.00 |
| 63. | Supplying, fitting and fixing cast iron ‘P’ or ‘S’ trap conforming to IS: 3989 / 1970 and 1729 / 1964 including lead caulked joints and painting two coats to the exposed surface. - ‘P’ Trap. - 100 mm. B) ‘S’ Trap. iii) 100 mm. | 4 6 | No. No. | 943.00 689.85 | 3772.00 4139.00 |
| 64. | Supplying, fitting and fixing C.I. round grating. i) 100 mm dia | 8 | No. | 58.00 | 464.00 |
| 65. | Orissa pattern water closet in white glazed vitreous chinaware of approved make supplied, fitted and fixed in position complete excluding ‘P’ or ‘S’ trap. i) 580 mm X 440 mm. | 1 | No. | 1660.00 | 1660.00 |
| 66. | Flat back urinal in white vitreous chinaware of approved make supplied, fitted and fixed in position with brass screws on 75 mm X 75 mm X 75 mm wooden blocks complete. iii) 465mm X355 mm X 265 mm. | 3 | No. | 766.00 | 2298.00 |
| 67. | Supplying, fitting and fixing32 mm dia flush pipe of approved brand. (a) C.P flush pipe. | 6 | No. | 276.00 | 1656.00 |
| 68. | Supplying porcelain partition wall of approved make of size 618 mm X 310 mm fitting and fixing the same complete in all respect. | 2 | No. | 1246.00 | 2492.00 |
| 69. | Supplying, fitting and fixing best quality Indian make mirror 5.5 mm thick with silvering as per I.S.I. specifications supported on fibre glass frame of any colour, frame size 550 mm X 400 mm. | 5 | No. | 488.00 | 2440.00 |
| 70. | Supplying, fitting and fixing towel rail with two brackets. (a) C.P. over brass. (i) 25 mm dia. and 750 mm long. | 5 | No. | 395.00 | 1975.00 |
| 71. | Supplying fitting and fixing approved brand PVC connector white flexible , with both ends coupling with heavy brass , CP nut 15mm diaiv)750 mm long | 10 | Each | 91.00 | 910.00 |
| 72. | Supplying fitting and fixing porcelain toilet paper holder of approved make with wooden spindle as necessary. - Box type size 150 mm X150mm | 4 | Each | 341.00 | 1364.00 |
| 73. | Supplying fitting and fixing porcelain soap tray of approved make. - Recessed soap tray of size 150mmX 150mm | 4 | Each | 375.00 | 1500.00 |
| 74. | Approved patern vent cowl conforming to Iss - 100 mm dia | 4 | Each | 317.00 | 1268.00 |
| 75 | Constructing Inspections pit of inside measurement 600 mm x 600 mm x upto 600 mm (depth) with 250 mm thick Ist class brick work in cement mortar (6:1) on all sides, bottom of the pit consisting of 100 mm thick cement concrete (6:3:1) with jhama khoa over a layer of jhama brick flat soling 12 mm thick (4:1) cement plaster to inside walls and out-side walls upto G.L. and 20 mm thick (4:1) plaster to bottom of the pit, providing necessary invert with cement concrete (4:2:1) with jhama chips 100 mm thick R.C.C. slab (4:2:1) with stone chips and necessary reinforcements upto 1% of approved make supplied, fitted and fixed in the earthwork in excavation in all sorts of soil, filling sides of the pit with earth and removing spoils after work complete in all respect. | 4 | No. | 5420.50 | 21682.00 |
| 76. | Construction of Septic tank of different capacities as per approved drawing with Ist class brick work in cement mortar (4:1) including two 560 mm dia R.C.C. manhole cover of approved make supplied, fitted and fixed in the top slab with necessary fittings, 19mm thick cement plaster (4:1) with neat cement finish to the internal surfaces and 12mm thick cement plaster (4:1) to outside wall upto 200 mm below G.I. floor finished with 25 mm thick gray artificial stone including supplying, fitting and fixing all necessary specials, fittings S.W. tees, C..I. foot rest etc. including excavating earth in all sorts of soil, shoring, bailing and pumping out water as necessary, up the chamber with clear water, removing foreign materials from the chamber and including constructing attached inspection pit as per approved drawing and connecting all necessary pipes, joints etc. with internal plaster work and artificial stone flooring is to be done with admixture of water proofing compound @ 5% by weight of cement , complete in all respect as per direction of Engineer-in-charge. v) For 20 users | 1 | No. | 36750.47 | 36750.00 |
| 77. | Construction of circular soak well 2.5 metre deep in all types of sandy soils with dry brick work upto 1.6 metre from the bottom having 150 mm intermediate cement brick work (6:1) band all round and cement brick work (6:1) upto 0.90 metre from top with 19 mm thick cement plastering (4:1) to inside face upto the depth of cement brick work, 12 mm thick cement plaster (4:1) on outer face from top of the well with brick metal (50 mm to 63 mm size) including R.C.C. cover slab of designed thickness with cement concrete (4:2:1) with stone chips with necessary reinforcement and shuttering including one 560 mm dia R.C.C. manhole cover of approved make supplied, fitted and fixed in the cover slab with necessary fittings, making necessary arrangements for pipe connections, excavation of well including shoring, dewatering and removing the excess earth from the premises as per direction complete in all respect with all costs of labour and materials. a) With 250 mm thick dry brick work and 250 mm thick cement brick work (6:1) and 1.00 m inside dia. | 1 | No. | 12259.54 | 12260.00 |
| Total for (B) Sanitary & Plumbing Works |  |  |  |  | 191123.00 |
| Total for (A) Building Works |  |  |  |  | 1450809.00 |
| Estimated Amount Total (A + B) |  |  |  |  | 1641932.00 |

Rs.  Sixteen lacks forty one thousand nine hundred thirty two only.